AIM: Simulate Linear Convolution and Circular convolution on discrete time signals

# LINEAR CONVOLUTION

# import numpy as np

# import matplotlib.pyplot as plt

# x = np.array([1, 2, 3, 4, 5])

# h = np.array([2, 4, 6, 8, 10])

# y = np.convolve(x, h)

# plt.stem(y)

# plt.title('Linear convolution')

# plt.xlabel('n')

# plt.ylabel('y[n]')

# plt.grid(True)

# plt.show()

# CIRCULAR CONVOLUTION

# import numpy as np

# import matplotlib.pyplot as plt

# x = np.array([1, 2, 3, 4, 5])

# h = np.array([2, 4, 6, 8, 10])

# N = len(x) + len(h) - 1

# X = np.fft.fft(x, N)

# H = np.fft.fft(h, N)

# y = np.fft.ifft(X * H)

# plt.stem(y.real)

# plt.title('Circular convolution')

# plt.xlabel('n')

# plt.ylabel('y[n]')

# plt.grid(True)

# plt.show()

# LINEAR AND CIRCULAR CONVOLUTION

# import numpy as np

# import matplotlib.pyplot as plt

# def linear_convolution(signal1, signal2):

# linear = np.convolve(signal1, signal2, mode='full')

# return linear

# def circular_convolution(signal1, signal2):

# fft_length = len(signal1) + len(signal2) - 1

# fft_signal1 = np.fft.fft(signal1, fft_length)

# fft_signal2 = np.fft.fft(signal2, fft_length)

# circular = np.fft.ifft(fft_signal1 * fft_signal2)

# return circular

# signal1 = np.array([1, 2, 3, 4, 5])

# signal2 = np.array([2, 4, 6, 8, 10])

# linear_con = linear_convolution(signal1, signal2)

# circular_con = circular_convolution(signal1, signal2)

# plt.figure(figsize=(10, 6))

# plt.subplot(2, 1, 1)

# plt.stem(linear_con)

# plt.title('Linear conv.')

# plt.xlabel('sample')

# plt.ylabel('amplitude')

# plt.subplot(2, 1, 2)

# plt.stem(circular_con.real)

# plt.title('Circular conv.')

# plt.xlabel('sample')

# plt.ylabel('amplitude')

# plt.tight_layout()

# plt.show()

# CONCLUSION

# In this experiment, we successfully simulated both linear and circular convolution of discrete-time signals using Python. The NumPy library was used to perform the convolution operations, while Matplotlib was used to visualize the resulting signals. By comparing the two outputs, we understood how linear and circular convolution differ and how convolution can be implemented practically for discrete-time signals.