Aim: Develop a program that performs audio convolution and inverse filtering to reduce or
suppress background music in a given song using different impulse responses.

Code:

# AIM: Audio Convolution and Inverse Filtering for Background Music Suppression

import numpy as np

import matplotlib.pyplot as plt

import scipy.signal as signal

import scipy.io.wavfile as wavfile

import librosa

def create_impulse_responses(sr):

delay_samples = int(0.05 * sr)

ir_echo = np.zeros(delay_samples + 1)

ir_echo[0] = 1.0

ir_echo[-1] = 0.5

t_ir = np.linspace(0, 0.2, int(0.2 * sr))

ir_reverb = np.exp(-15 * t_ir) * np.random.randn(len(t_ir))

ir_reverb[0] = 1.0

ir_lowpass = signal.firwin(101, cutoff=2500, fs=sr)

return {

"1_Echo_Delay": ir_echo,

"2_Room_Reverberation": ir_reverb,

"3_LowPass_Dampening": ir_lowpass

}

def regularized_inverse_filter(h, epsilon=1e-2, n_fft=262144):

H = np.fft.rfft(h, n=n_fft)

G = np.conj(H) / (np.abs(H)**2 + epsilon)

g = np.fft.irfft(G, n=n_fft)

return np.real(g)

def run_audio_experiment(audio_path):

print(f"Loading MP3 audio file: {audio_path}...")

audio_data, sr = librosa.load(audio_path, sr=None, mono=True, duration=10.0)

audio_data = audio_data / np.max(np.abs(audio_data))

impulse_responses = create_impulse_responses(sr)

plt.figure(figsize=(12, 10))

plt.subplot(4, 1, 1)

plt.plot(audio_data, color='k', alpha=0.7)

plt.title("Original MP3 Audio Track")

plt.ylabel("Amplitude")

plt.grid(True)

plot_idx = 2

for name, ir in impulse_responses.items():

print(f"Processing IR: {name}...")

convolved_audio = signal.fftconvolve(audio_data, ir, mode='same')

inv_filter = regularized_inverse_filter(ir, epsilon=1e-2)

filtered_audio = signal.fftconvolve(convolved_audio, inv_filter, mode='same')

filtered_audio = filtered_audio[:len(audio_data)]

output_filename = f"output_{name}.wav"

wavfile.write(output_filename, sr, (filtered_audio * 32767).astype(np.int16))

print(f"  -> Processed track saved to: {output_filename}")

plt.subplot(4, 1, plot_idx)

plt.plot(filtered_audio, alpha=0.8)

plt.title(f"Inverse Filtered Output ({name})")

plt.ylabel("Amplitude")

plt.grid(True)

plot_idx += 1

plt.xlabel("Sample Index")

plt.tight_layout()

plt.show()

run_audio_experiment('song.mp3')

Output:

Conclusion :

In this experiment, audio convolution and regularized inverse filtering were applied to an audio track (.mp3) across multiple synthetic impulse responses (Echo/Delay, Room Reverberation, and Low-Pass Dampening).

The results demonstrate that inverse filtering successfully restores or equalizes acoustic channel distortions - such as suppressing discrete echoes or reducing room reverberation tails - when the system's impulse response is known. However, the experiment highlights a fundamental limitation of linear filtering: inverse filtering acts as a global system equalizer and cannot perform blind source separation (such as isolating vocals/lyrics from digital background music) because both components share overlapping frequency bands within the same composite waveform.

Github Link: https://github.com/divaparekh06/Digital-Signal-And-Image-Processing.git